Distribution Information sheet
Census division Base Maps 1851-1961
HACOLP_cds_1851.shp  every decade up to  HACOLP_cds_1961.shp

CDs  based on original linework from  the National Atlas of Canada 1:7 500 000 data set.

Compiled partially for the Historical Atlas of Canada; completed for the Historical Atlas of Canada Online Learning Project. See:

http://www.historicalatlas.ca/website/hacolp/national_perspectives/population/UNIT_25/index.htm

Historical data:

Censuses of Canada. For individual sources see:

http://www.historicalatlas.ca/website/hacolp/national_perspectives/population/UNIT_25/U25_sources.htm

also see:

http://www.collectionscanada.gc.ca/census/index-e.html#databases

http://www.collectionscanada.gc.ca/databases/census-1881/index-e.html

http://www.collectionscanada.gc.ca/databases/census-1891/index-e.html

http://www.collectionscanada.gc.ca/databases/census-1901/index-e.html

1861

Census of 1861 online for Lower Canada and Upper Canada

For NB, NS and Territories

Census of 1871 online for Canada: Table V, Population of 1861 and 1871 compared by Electoral Districts

GIS data: National Atlas of Canada, Atlas of Canada Base maps, 1:7,500,000 and 1:2 000 000 layers.

National Atlas of Canada, National Scale Frameworks, 1:1 000 000 layers.

See Geogratis website: http://www.geogratis.cgdi.gc.ca/geogratis/en/download/atlas.html

Nominal scale : 		1:7 500 000

Type of Data : 		Vector

Spatial Reference: 	Geographic coordinate system-GCS_North American 1927

Map Projection-North American 1927 Lambert Conformal Conic

Standard Parallels: 49º N, 77º N; Central Meridian: 95º W.; Latitude origin: 49º N; Units: meters

Format : 		ESRI Shape File

Attribute Fields:

CDNAME		text 30		CD name only

CDPROV		text 10		Province code

PROVNAME		text 30		Province name

CDNAMEPROV	text 30		CD_Prov concatenation; unique identifier

CDPOP			long integer 	Population

CDAREASQKM	double		Land area in square km

( Calculate: [Shape_Area] / 1000000 )

CDPOPDENS		double		Population density in persons per sq km

( Calculate: [CDPOP] / [CDAREASQKM] )