Identification

Description:

Abstract:

Western Grains Transition Payment Program (WGTPP) Generalized Prairie Landcover is in seamless vector format within the extent of Western Canada.  Landcover includes cropland, forage, grassland, etc.

The Landcover Generalization process was undertaken to solve rendering problems of the original vectorized landcover data due to its unwieldy/overwhelming size.  LANDSAT 7 imagery used in the process was collected during the WGTPP.  This landcover imagery has a 30 meter resolution and is stored in over 1,100 vectorized 1:50,000 map sheet tiles. The data requires over seven gigabytes of disc space.  If the user wishes to view a specific area, they must identify the 1:50,000 map sheet, then browse to the desired mapsheet. Due to the great detail, this vector product (l02 in GIS_LIB) becomes computationally impossible on the desktop computer when multiple mapsheets are drawn. Spatial analysis and modeling at provincial or western Canada extents are also impossible in the mapsheet format.

The WGTPP Generalized Landcover is a seamless vector coverage, covering the agricultural extent of Western. The generalized vector coverage is 124 MB in size, and requires roughly 60 times less space than the mapsheet tiles, thus making it easily used with desktop GIS applications.  An algorithm was created using the processing tools of the ArcINFO GRID module. A series of AMLs and digital rules have been implemented in the event that new satellite imagery is acquired.

Purpose:

To make the prairie landcover available for analysis purposes and large scale mapping.

There were 6 goals for the Generalized Landcover:

·To be used cartographically with outputs at a scale of 1:1 million.

·As a viewing and analytical tool within PFRA regions and districts

·A planning tool, and a 'Quick' statistical tool for analytical staff

·To be incorporated into Internet Mapping applications

·To develop and establish a process with digital rules of generalization for use with new satellite imagery

·Collaborating in partnership with various levels of government, the private sector and the academic community to capitalize on their collective expertise and to ensure seamless public delivery of geospatial information.

Supplemental Information:

For all associated information including technical papers, presentations etc download data at ???? or contact James Ashton ashtonj@agr.gc.ca

Constraints:

Access Constraints: Data are subject to the AAFC Unrestricted Use License Agreement

Use Constraints: Data are subject to the AAFC Unrestricted Use License Agreement

Data Set Contact:

Person: James Ashton

Organization: Agriculture and Agri - Food Canada

Email: ashtonj@agr.gc.ca

Citation:

Title: Western Grains Transition Payment Program (WGTPP) Generalized Landcover (poly)

Originator: Agriculture and Agri - Food Canada

Publisher: Agriculture and Agri - Food Canada

Publication Date: Known Year 2001

Time Period of Content:

Currentness Reference: Publication date

Date Information: Range of Dates: Beginning Date: Year 1993 to Year 1995

Status:

Progress: complete

Update Frequency: none planned

Keywords:

Theme: generalized, landcover, land use, cropland, forage, grassland, shrubs, trees, wetlands, waterbodies, other lands, mud, sand, saline

Data Quality

General:

Logical Consistency Report: Please refer to document: PFRA Generalized Landcover Technical Paper

Completeness Report: Please refer to document: PFRA Generalized Landcover Technical Paper

Process Description: Please refer to document: PFRA Generalized Landcover Technical Paper

Source Information:

Source Title: Western Grains Transition Payment Program (WGTPP) Generalized Landcover (poly)

Source Originator:  Agriculture and Agri - Food Canada

Other Source Citation Details:

Source Scale Denominator: 50,000

Source Publication Date: Known Year 1995

Source Title: Landsat 7 Satellite Imagery

Source Originator:  Department of Natural Resources Canada

Other Source Citation Details: "© Department of Natural Resources Canada. All rights reserved."

Source Scale Denominator: 50,000

Source Publication Date: Known Year 1995

Attributes

Field Name: Grid Code

Field Definition: Grid code carried over from the raster which was generalized before vector conversion.

Definition Source: Original Raster Grids

Values do not exist in a known, predefined set

Field Name: Class

Field Definition: Classification of land cover

Definition Source: Agriculture and Agri-Food Canada

List of Values:

Cropland

Forage

Grassland

Mud/Sand/Saline

Other Lands

Shrubs

Trees

Waterbodies

Wetlands

Field Name: Acres

Field Definition: Acres of Land

Definition Source: Agriculture and Agri-Food Canada

Values do not exist in a known, predefined set

Description: Calculated using the area field.