Station Data Catalogue  - Explanation of Tabulated Data

#### Station Number

The 7 digit number is the permanent identifier of a site at which official weather observations have been taken, and is also used to identify data from the site when transferred to electronic media for processing. The first digit assigned, identifies the province, the second and third digits identify the climatological district within the province, and the final four digits are assigned so that when they are arranged in ascending order the station names are in alphabetical order. Occasionally different names have been used for the same site and in these cases the data may be listed under different numbers with a cross reference, and are indicated by "+" in front-of the name. A change in name may occur with the number remaining the same and this is also indicated by "+" in front of the name. When observations are discontinued at a site, the number is not used for subsequent stations (which may, or may not, differ in name) unless it is judged that the records from the earlier and subsequent stations may be combined for most climatological purposes. This does not mean that data from sites with different station names and numbers may not be combined for specific purposes, but such combinations should be done at the discretion of the user.

#### Latitude, Longitude And Elevation

The latitude and longitude of each site is given to the nearest minute. These locations are generally for the instrument site; however prior to April 1, 1986 at principal stations (airports) the locations given were normally the official airport locations. The accuracy of these locations depended on the quality of the reference maps available at the time. The elevation of each site is given to the nearest metre and is generally the height of ground on which the instruments are exposed. Prior to April 1, 1986 at principal stations the elevation was generally the established elevation which is equal to the aerodrome elevation at airports or to the elevation determined for the barometer cistern at nonairport principal stations.

- Periods

The year and month (January - 01, February - 02, etc.) that the program began and ended.

* AES has summary information only. Further data may be available from the Quebec Meteorological Service (appears in Quebec volume only). --- data not processed or dates unknown.

| S | program in operation only during summer months |
| --- | --- |
| W | program in operation only during winter months |

- Observing Program

Entries in these columns give information on observing programs at individual stations and the applicable date for each program. A new line is listed for an observing station whenever:

- there is a charge in observing program

- observations are resumed after no reports have been received from a particular program for a period greater than 3 months. Note exception is made to this rule for those stations taking observations during the summer or the winter months only (indicated by S or W respectively).

- there has been a change in station name. If the change in name also involves a change in station number, then the information for the old name is transferred to the new number and the old name is listed only for the period of time that the name was valid.

- there has been a minor change in site location. In many cases there will be minor changes in the coordinates or elevation. A minor change in the site may be listed as a new line to alert the user to a possible discontinuity in the record.

- Hourly Weather

Normally includes observations of sky conditions, visibility (miles), weather and obstructions to vision, atmospheric pressure (kPa), temperature (C), humidity (%); wind speed (kts) and direction (tens of degrees); cloud amount (tenths) and/or obscuring phenomena (tenths).

| X | 24 hours per day |
| --- | --- |
| B | 8 observations per day, every 3 hours |
| C | 4 observations per day, every 6 hours |
| D | irregular observations, daily |
| E | 8 observations per day, every 3 hours plus extra hours |
| F | 4 observations per day, every 6 hours plus extra hours |
| G | Automatic station (various types) irregular, daily |
| H | Automatic station (various types) 24 hours per day |
| J | irregular observations - not processed |

- Temperature

| X | Daily readings of maximum and minimum temperature (C). |
| --- | --- |

- Precipitation

| X | Daily values of liquid, freezing or frozen precipitation (drizzle, rain, snow, snow pellets, snow grains, ice pellets, hail and ice crystals) in mm. |
| --- | --- |

- Rate Of Precipitation

| X | Tipping bucket rain gauge, hourly rainfall values and rate of rainfall in mm. |
| --- | --- |
| S | Fischer and Porter precipitation gauge, quarter hourly values and rate of precipitation in mm. |
| B | Tipping bucket rain gauge, hourly rainfall values and rate of rainfall in mm and Fischer and Porter precipitation gauge, quarter hourly values and rate of precipitation in mm. |
| W | Weighing type precipitation gauge (mm) |
| V | Volumetric precipitation gauge, periodic measurements in mm. |

- Wind

Note: Most stations taking 24 hourly weather observations record hourly - one minute mean wind speed and direction from U2A dial. This type of wind program is not shown under "wind" in the Observing Program. Many hourly stations taking less than 24 observations per day complete a daily (24 observations) wind program from autographic recorders. This program is also indicated by a "U".

| B | Data processed from 45B autographic record, hourly total wind run in km/h and direction to 8 compass points. |
| --- | --- |
| U | Data processed from U2A autographic record, hourly (shortduration mean) wind speed in km/h and direction to tens of degrees. |

- Soil Temperature

| D | Morning values recorded for depths: 5, 10, 20, 50, 100, 150, and 300 cm in |
| --- | --- |
| C | and afternoon values recorded for the first three depths only. |
| G | Same as D but from automatic recorder. |

- Evaporation

Net water loss from pan and calculated lake evaporation in mm.

| A | Type A pan, daily values. |
| --- | --- |
| R | Type A pan, daily values using radioactive tracer (for Atomic Energy of Canada). |

- Sunshine

| R | Hourly values of bright sunshine from an electronic recorder |
| --- | --- |
| S | Hourly values of bright sunshine |

- Radiation

Radiation values are recorded hourly in Local Apparent Time and the units are expressed in megajoules per square metre, except daylight illumination is in 1000 lumen-hours per square metre.

| A | Global solar radiation RF1 |
| --- | --- |
| B | Sky radiation RF2 |
| C | Reflected solar radiation RF3 |
| D | Net radiation RF4 |
| E | Daylight illumination RF7 |
| F | Global solar radiation RF1 and sky radiation RF2 |
| G | Global solar radiation RF1 and Reflected solar radiation RF3 |
| H | Global solar radiation RF1 and Net radiation RF4 |
| J | Global solar radiation RF1, Sky radiation RF2 and Reflected solar radiation RF3 |
| K | Global solar radiation RF1, Sky radiation RF2, Reflected solar radiation RF3 and Net radiation RF4 |
| L | Global solar radiation RF1, Reflected solar radiation RF3 and Net radiation RF4 |
| M | Global solar radiation RF1, Sky radiation RF2, Reflected solar radiation RF3, Net radiation RF4 and Daylight illumination RF7 |
| N | Global solar radiation RF1, Sky radiation RF2 and Net radiation RF4 |

- Ozone

| T | Total ozone and umkehr observations using Dobson spectrophotometer, daily in m-atm-cm and ?mb units respectively |
| --- | --- |
| S | Ozonesonde on a weekly basis (usually Wednesday) in ?mb units |
| O | Total ozone and umkehr observations using Dobson spectrophotometer, daily in m-atm-cm and ?mb units respectively and Ozonesonde on a weekly basis (usually Wednesday) in ?mb units |

- Upper Air

Readings of pressure (kPa), altitude (m), air temperature (C), relative humidity (%), wind speed (m/s), and wind direction (degrees) for standard pressure surfaces at 00 and 12 UTC.

| X | Rawinsonde (temperature, pressure, humidity and wind) |
| --- | --- |
| W | Rawin (wind only) |
| T | Radiosonde (temperature, pressure and humidity) |

- Snow Survey

Snow surveys by designated stations are made at regular intervals during the winter months to determine the water equivalent (mm) and depth of the snow pack (cm).

| V | 5 points (30 m apart), measurements taken on the lst, 8th, 15th and 23rd of each month. |
| --- | --- |
| X | 10 points (30 m apart), measurements taken on the lst and 15th days of each month. |

- Air Quality

| T | Turbidity, 3 observations per day at 09, 12, and 15 LST during cloudless days giving a measurement of the total aerosol content (suspended solid and liquid particles other than clouds) in a column of the atmosphere from the observer to the sun. |
| --- | --- |
| A | Canadian Air and Precipitation Monitoring Network, CAPMoN. |
| C | precipitation chemistry |
| B | Turbidity, 3 observations per day at 09, 12, and 15 LST during cloudless days giving a measurement of the total aerosol content (suspended solid and liquid particles other than clouds) in a column of the atmosphere from the observer to the sun and precipitation chemistry |
| P | particulate sampling |
| G | gaseous sampling |
| H | Turbidity, 3 observations per day at 09, 12, and 15 LST during cloudless days giving a measurement of the total aerosol content (suspended solid and liquid particles other than clouds) in a column of the atmosphere from the observer to the sun, precipitation chemistry and particulate sampling |
| J | Turbidity, 3 observations per day at 09, 12, and 15 LST during cloudless days giving a measurement of the total aerosol content (suspended solid and liquid particles other than clouds) in a column of the atmosphere from the observer to the sun, precipitation chemistry, particulate sampling and gaseous sampling |

- Nipher

| N | nipher snow measurement in mm water equivalent |
| --- | --- |

- Region

| P | Pacific |
| --- | --- |
| W | Western |
| C | Central |
| O | Ontario |
| Q | Quebec |
| A | Atlantic |
| D | Department of National Defence |
| H | AES Downsview |
| N | Northern |

- Other

| @ | May appear in any column, indicating incomplete information at time of printing |
| --- | --- |